Complete Threat-to-Control Mapping

AI-Enabled CGM Security Framework

Supplementary Material

for

"A Security Framework for AI-Enabled Continuous Glucose Monitoring: Translating FDA Guidance into Actionable Controls"

Venkata Sai Abhinav Piratla, Sajal Bhatia, and Sahaj Saxena

CyberAI 2026 Conference

27 Threats × 38 FDA Requirements × MITRE ATT&CK × NIST SP 800-53 Rev. 5

1. Complete Threat Catalog with Full Mappings

Each threat entry below provides the complete mapping chain: threat description, MITRE ATT&CK techniques, FDA requirement mappings with relevance scores, NIST SP 800-53 Rev. 5 controls, and implementation priority assessment.

1.1. Control-Theoretic Threats (4)

T-001: Glucose-Insulin Model Manipulation

Category: Control-Theoretic (CT)  |  Severity: Critical  |  Primary Layer: 4

Adversary manipulates physiological glucose-insulin model parameters used by closed-loop controllers, causing systematically incorrect insulin delivery calculations.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T0836 | Modify Parameter | ICS |
| T0855 | Unauthorized Command Message | ICS |

NIST SP 800-53 Rev. 5 Controls: SI-7, SA-11, CA-2, AU-2

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-006 | Security Risk Assessment with Traceability | 2 (Direct) | Cybersec. Section V-A |
| CYB-019 | Code/Data/Execution Integrity | 2 (Direct) | Cybersec. Appendix 1-D |
| AI-003 | AI Risk Assessment | 2 (Direct) | AI-DSF Section VII |
| AI-005 | AI Performance Validation | 1 (Indirect) | AI-DSF Section IX |
| CYB-024 | Cybersecurity Testing Evidence | 1 (Indirect) | Cybersec. Section VII-A |

Priority: S=5, L=2, D=3, RPN=30, C=4 → Q4

T-002: MPC Model Inaccuracy

Category: Control-Theoretic (CT)  |  Severity: High  |  Primary Layer: 4

Inaccurate glucose-insulin dynamics in Model Predictive Control (MPC) controllers lead to dangerous insulin delivery due to parameter estimation errors or deliberate manipulation.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T0836 | Modify Parameter | ICS |
| T0880 | Loss of Safety | ICS |

NIST SP 800-53 Rev. 5 Controls: SI-7, SA-11, CA-8, AU-6

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-006 | Security Risk Assessment with Traceability | 2 (Direct) | Cybersec. Section V-A |
| CYB-019 | Code/Data/Execution Integrity | 1 (Indirect) | Cybersec. Appendix 1-D |
| AI-003 | AI Risk Assessment | 2 (Direct) | AI-DSF Section VII |
| AI-005 | AI Performance Validation | 2 (Direct) | AI-DSF Section IX |
| AI-007 | AI Performance Monitoring | 1 (Indirect) | AI-DSF Section XI |

Priority: S=5, L=3, D=3, RPN=45, C=4 → Q2

T-003: Setpoint Manipulation

Category: Control-Theoretic (CT)  |  Severity: Critical  |  Primary Layer: 3

Adversary alters target glucose setpoint to unsafe levels, causing the control algorithm to drive blood glucose to hypoglycemic or hyperglycemic ranges.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T0836 | Modify Parameter | ICS |
| T0831 | Manipulation of Control | ICS |

NIST SP 800-53 Rev. 5 Controls: AC-3, AC-6, SI-7, AU-2, IA-2

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-016 | Authentication Controls | 2 (Direct) | Cybersec. Appendix 1-A |
| CYB-017 | Authorization and Access Controls | 2 (Direct) | Cybersec. Appendix 1-A |
| CYB-019 | Code/Data/Execution Integrity | 2 (Direct) | Cybersec. Appendix 1-D |
| CYB-006 | Security Risk Assessment with Traceability | 1 (Indirect) | Cybersec. Section V-A |
| CYB-024 | Cybersecurity Testing Evidence | 1 (Indirect) | Cybersec. Section VII-A |

Priority: S=5, L=3, D=2, RPN=30, C=3 → Q3

T-004: Control Algorithm Exploitation

Category: Control-Theoretic (CT)  |  Severity: High  |  Primary Layer: 4

Exploiting vulnerabilities in PID/MPC/RL control algorithms through crafted inputs that cause oscillatory or divergent insulin delivery behavior.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T0836 | Modify Parameter | ICS |
| T0882 | Theft of Operational Information | ICS |

NIST SP 800-53 Rev. 5 Controls: SA-11, SI-7, CA-8, SC-7

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-019 | Code/Data/Execution Integrity | 2 (Direct) | Cybersec. Appendix 1-D |
| AI-003 | AI Risk Assessment | 2 (Direct) | AI-DSF Section VII |
| AI-005 | AI Performance Validation | 2 (Direct) | AI-DSF Section IX |
| CYB-006 | Security Risk Assessment with Traceability | 1 (Indirect) | Cybersec. Section V-A |
| CYB-008 | Security by Design Principles | 1 (Indirect) | Cybersec. Section V-C |

Priority: S=4, L=2, D=4, RPN=32, C=5 → Q4

1.2. Software Threats (8)

T-005: Cloud API Exploitation

Category: Software (SW)  |  Severity: High  |  Primary Layer: 2

Exploiting vulnerabilities in cloud service APIs to gain unauthorized access to patient glucose data, modify dosing parameters, or disrupt cloud-based analytics.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T1190 | Exploit Public-Facing Application | Enterprise |
| T0883 | Unauthorized Access | ICS |

NIST SP 800-53 Rev. 5 Controls: SC-7, AC-4, SI-3, RA-5, SA-11

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-016 | Authentication Controls | 2 (Direct) | Cybersec. Appendix 1-A |
| CYB-019 | Code/Data/Execution Integrity | 2 (Direct) | Cybersec. Appendix 1-D |
| CYB-024 | Cybersecurity Testing Evidence | 2 (Direct) | Cybersec. Section VII-A |
| CYB-009 | Defense-in-Depth Architecture | 1 (Indirect) | Cybersec. Section V-D |
| CYB-020 | Data Confidentiality and Privacy | 1 (Indirect) | Cybersec. Appendix 1-E |

Priority: S=4, L=3, D=3, RPN=36, C=3 → Q3

T-006: Insufficient Authentication

Category: Software (SW)  |  Severity: Critical  |  Primary Layer: 3

Unauthorized device connections and command injection due to weak, default, or missing authentication mechanisms, as demonstrated in the 2019 Medtronic MiniMed advisory.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T1078 | Valid Accounts | Enterprise |
| T0812 | Default Credentials | ICS |

NIST SP 800-53 Rev. 5 Controls: IA-2, IA-5, AC-7, AC-14, IA-3

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-016 | Authentication Controls | 2 (Direct) | Cybersec. Appendix 1-A |
| CYB-006 | Security Risk Assessment with Traceability | 2 (Direct) | Cybersec. Section V-A |
| CYB-024 | Cybersecurity Testing Evidence | 2 (Direct) | Cybersec. Section VII-A |
| CYB-017 | Authorization and Access Controls | 1 (Indirect) | Cybersec. Appendix 1-A |
| CYB-008 | Security by Design Principles | 1 (Indirect) | Cybersec. Section V-C |

Priority: S=5, L=5, D=3, RPN=75, C=2 → Q1

T-007: Firmware Code Injection

Category: Software (SW)  |  Severity: Critical  |  Primary Layer: 1

Malicious code injection to alter device behavior by replacing or modifying firmware, enabling persistent compromise of insulin pump or CGM controller functionality.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T1542 | Pre-OS Boot | Enterprise |
| T0857 | System Firmware | ICS |

NIST SP 800-53 Rev. 5 Controls: CM-2, CM-3, SI-7, SA-10, SC-34

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-019 | Code/Data/Execution Integrity | 2 (Direct) | Cybersec. Appendix 1-D |
| CYB-023 | Vulnerability Patching Capability | 2 (Direct) | FDORA 524B(b)(2) |
| CYB-011 | Software Bill of Materials (SBOM) | 2 (Direct) | FDORA 524B(b)(3) |
| CYB-012 | Secure Firmware Update Mechanism | 2 (Direct) | Cybersec. Section VI-A |
| CYB-006 | Security Risk Assessment with Traceability | 1 (Indirect) | Cybersec. Section V-A |

Priority: S=5, L=3, D=4, RPN=60, C=4 → Q2

T-008: Session Hijacking

Category: Software (SW)  |  Severity: High  |  Primary Layer: 3

Hijacking authenticated sessions between CGM system components (sensor-controller, controller-pump, device-cloud) to issue unauthorized commands.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T1563 | Remote Service Session Hijacking | Enterprise |
| T0830 | Manipulation of View | ICS |

NIST SP 800-53 Rev. 5 Controls: SC-23, AC-12, IA-11, AU-2

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-016 | Authentication Controls | 2 (Direct) | Cybersec. Appendix 1-A |
| CYB-018 | Cryptographic Controls | 1 (Indirect) | Cybersec. Appendix 1-C |
| CYB-019 | Code/Data/Execution Integrity | 1 (Indirect) | Cybersec. Appendix 1-D |
| CYB-014 | Secure Communication Protocols | 1 (Indirect) | Cybersec. Section VI-C |

Priority: S=4, L=3, D=3, RPN=36, C=3 → Q3

T-009: Replay Attacks

Category: Software (SW)  |  Severity: High  |  Primary Layer: 2

Retransmitting previously valid CGM readings or insulin commands to cause inappropriate insulin delivery or mask actual glucose levels.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T1557 | Adversary-in-the-Middle | Enterprise |
| T0830 | Manipulation of View | ICS |

NIST SP 800-53 Rev. 5 Controls: SC-23, AU-2, SC-13, SC-8

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-018 | Cryptographic Controls | 2 (Direct) | Cybersec. Appendix 1-C |
| CYB-019 | Code/Data/Execution Integrity | 2 (Direct) | Cybersec. Appendix 1-D |
| CYB-014 | Secure Communication Protocols | 1 (Indirect) | Cybersec. Section VI-C |
| CYB-021 | Event Detection and Logging | 1 (Indirect) | Cybersec. Appendix 1-F |

Priority: S=4, L=4, D=3, RPN=48, C=2 → Q1

T-010: Denial of Service (DoS)

Category: Software (SW)  |  Severity: High  |  Primary Layer: 2

Overwhelming device communication channels or cloud resources to disrupt real-time glucose monitoring and insulin delivery control loops.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T1498 | Network Denial of Service | Enterprise |
| T0814 | Denial of Service | ICS |

NIST SP 800-53 Rev. 5 Controls: SC-5, CP-7, SI-4, SC-7

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-022 | Resiliency and Recovery | 2 (Direct) | Cybersec. Appendix 1-G |
| CYB-014 | Secure Communication Protocols | 2 (Direct) | Cybersec. Section VI-C |
| CYB-009 | Defense-in-Depth Architecture | 1 (Indirect) | Cybersec. Section V-D |
| CYB-008 | Security by Design Principles | 1 (Indirect) | Cybersec. Section V-C |

Priority: S=4, L=4, D=3, RPN=48, C=1 → Q1

T-011: Man-in-the-Middle (MITM) Attacks

Category: Software (SW)  |  Severity: Critical  |  Primary Layer: 2

Intercepting and altering data in transit between CGM sensor, controller, insulin pump, and cloud services to modify glucose readings or insulin commands.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T1557 | Adversary-in-the-Middle | Enterprise |
| T0830 | Manipulation of View | ICS |

NIST SP 800-53 Rev. 5 Controls: SC-8, SC-12, SC-23, SC-13

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-018 | Cryptographic Controls | 2 (Direct) | Cybersec. Appendix 1-C |
| CYB-014 | Secure Communication Protocols | 2 (Direct) | Cybersec. Section VI-C |
| CYB-019 | Code/Data/Execution Integrity | 2 (Direct) | Cybersec. Appendix 1-D |
| CYB-021 | Event Detection and Logging | 1 (Indirect) | Cybersec. Appendix 1-F |

Priority: S=5, L=3, D=3, RPN=45, C=4 → Q2

T-012: Privilege Escalation

Category: Software (SW)  |  Severity: High  |  Primary Layer: 3

Escalating from limited user access to administrative or clinical-level privileges on the CGM mobile application or cloud management portal.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T1068 | Exploitation for Privilege Escalation | Enterprise |
| T0890 | Exploitation for Privilege Escalation | ICS |

NIST SP 800-53 Rev. 5 Controls: AC-6, AC-5, AU-2, SI-7, CM-7

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-017 | Authorization and Access Controls | 2 (Direct) | Cybersec. Appendix 1-A |
| CYB-016 | Authentication Controls | 2 (Direct) | Cybersec. Appendix 1-A |
| CYB-024 | Cybersecurity Testing Evidence | 1 (Indirect) | Cybersec. Section VII-A |
| CYB-010 | Least Privilege Implementation | 2 (Direct) | Cybersec. Section V-E |

Priority: S=4, L=3, D=3, RPN=36, C=2 → Q3

1.3. Hardware Threats (5)

T-013: CGM Sensor Failures

Category: Hardware (HW)  |  Severity: High  |  Primary Layer: 1

Signal loss or calibration drift disrupting closed-loop glucose control, including premature sensor failures leading to loss of glucose monitoring data.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T0814 | Denial of Service | ICS |
| T0826 | Loss of Availability | ICS |

NIST SP 800-53 Rev. 5 Controls: SI-4, PE-14, CP-2, MA-2

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-022 | Resiliency and Recovery | 2 (Direct) | Cybersec. Appendix 1-G |
| CYB-006 | Security Risk Assessment with Traceability | 1 (Indirect) | Cybersec. Section V-A |
| CYB-028 | Postmarket Cybersecurity Monitoring | 1 (Indirect) | Cybersec. Section VIII-B |
| CYB-024 | Cybersecurity Testing Evidence | 1 (Indirect) | Cybersec. Section VII-A |

Priority: S=4, L=4, D=2, RPN=32, C=3 → Q3

T-014: Sensor Calibration Manipulation

Category: Hardware (HW)  |  Severity: Critical  |  Primary Layer: 1

Deliberately altering CGM sensor calibration parameters to produce systematically inaccurate glucose readings, leading to incorrect insulin dosing.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T0836 | Modify Parameter | ICS |
| T0885 | Loss of Engineering Control | ICS |

NIST SP 800-53 Rev. 5 Controls: SI-7, PE-3, AU-2, CM-3

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-019 | Code/Data/Execution Integrity | 2 (Direct) | Cybersec. Appendix 1-D |
| CYB-015 | Physical Security Controls | 1 (Indirect) | Cybersec. Section VI-D |
| CYB-006 | Security Risk Assessment with Traceability | 1 (Indirect) | Cybersec. Section V-A |
| CYB-021 | Event Detection and Logging | 1 (Indirect) | Cybersec. Appendix 1-F |

Priority: S=5, L=2, D=3, RPN=30, C=4 → Q4

T-015: Physical Tampering

Category: Hardware (HW)  |  Severity: High  |  Primary Layer: 1

Physical access enables settings modification, data extraction, firmware dumping, or hardware component replacement on CGM/pump devices.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T0879 | Damage to Property | ICS |
| T0831 | Manipulation of Control | ICS |

NIST SP 800-53 Rev. 5 Controls: PE-3, PE-6, MP-4, PE-4

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-015 | Physical Security Controls | 2 (Direct) | Cybersec. Section VI-D |
| CYB-019 | Code/Data/Execution Integrity | 1 (Indirect) | Cybersec. Appendix 1-D |
| CYB-006 | Security Risk Assessment with Traceability | 1 (Indirect) | Cybersec. Section V-A |
| CYB-008 | Security by Design Principles | 1 (Indirect) | Cybersec. Section V-C |

Priority: S=3, L=2, D=2, RPN=12, C=3 → Q3

T-016: Battery/Power Depletion

Category: Hardware (HW)  |  Severity: Medium  |  Primary Layer: 1

Deliberately draining device battery through repeated unnecessary BLE connection requests or computation-intensive operations to disable glucose monitoring.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T0826 | Loss of Availability | ICS |
| T0814 | Denial of Service | ICS |

NIST SP 800-53 Rev. 5 Controls: PE-11, CP-2, SI-4, SC-5

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-022 | Resiliency and Recovery | 2 (Direct) | Cybersec. Appendix 1-G |
| CYB-009 | Defense-in-Depth Architecture | 1 (Indirect) | Cybersec. Section V-D |
| CYB-008 | Security by Design Principles | 1 (Indirect) | Cybersec. Section V-C |

Priority: S=3, L=3, D=2, RPN=18, C=4 → Q4

T-017: Hardware Supply Chain Compromise

Category: Hardware (HW)  |  Severity: High  |  Primary Layer: 1

Compromised hardware components introduced during manufacturing or distribution, including counterfeit sensors or tampered microcontrollers with backdoor access.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T1195 | Supply Chain Compromise | Enterprise |
| T0862 | Supply Chain Compromise | ICS |

NIST SP 800-53 Rev. 5 Controls: SR-3, SR-5, SR-11, SA-12, PE-3

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-005 | Third-Party Component Security | 2 (Direct) | Cybersec. Section IV-D |
| CYB-011 | Software Bill of Materials (SBOM) | 2 (Direct) | FDORA 524B(b)(3) |
| CYB-006 | Security Risk Assessment with Traceability | 1 (Indirect) | Cybersec. Section V-A |
| CYB-019 | Code/Data/Execution Integrity | 1 (Indirect) | Cybersec. Appendix 1-D |

Priority: S=4, L=2, D=4, RPN=32, C=5 → Q4

1.4. Communication Threats (6)

T-018: BLE Eavesdropping

Category: Communication (COM)  |  Severity: High  |  Primary Layer: 2

Passive interception of Bluetooth Low Energy communications between CGM sensor and controller to capture glucose readings, device identifiers, and protocol details.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T1040 | Network Sniffing | Enterprise |
| T0842 | Network Sniffing | ICS |

NIST SP 800-53 Rev. 5 Controls: SC-8, SC-13, AC-18, SC-40

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-018 | Cryptographic Controls | 2 (Direct) | Cybersec. Appendix 1-C |
| CYB-020 | Data Confidentiality and Privacy | 2 (Direct) | Cybersec. Appendix 1-E |
| CYB-014 | Secure Communication Protocols | 2 (Direct) | Cybersec. Section VI-C |
| CYB-008 | Security by Design Principles | 1 (Indirect) | Cybersec. Section V-C |

Priority: S=3, L=4, D=2, RPN=24, C=2 → Q3

T-019: BLE Pairing Exploitation

Category: Communication (COM)  |  Severity: Critical  |  Primary Layer: 2

Exploiting BLE pairing vulnerabilities (e.g., SweynTooth, KNOB attack) to establish unauthorized connections with CGM devices and inject commands.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T0860 | Wireless Compromise | ICS |
| T0812 | Default Credentials | ICS |

NIST SP 800-53 Rev. 5 Controls: IA-3, SC-8, AC-18, SC-13

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-016 | Authentication Controls | 2 (Direct) | Cybersec. Appendix 1-A |
| CYB-018 | Cryptographic Controls | 2 (Direct) | Cybersec. Appendix 1-C |
| CYB-014 | Secure Communication Protocols | 2 (Direct) | Cybersec. Section VI-C |
| CYB-024 | Cybersecurity Testing Evidence | 1 (Indirect) | Cybersec. Section VII-A |

Priority: S=4, L=4, D=3, RPN=48, C=3 → Q1

T-020: Wireless Replay

Category: Communication (COM)  |  Severity: High  |  Primary Layer: 2

Capturing and retransmitting valid BLE/RF messages to re-issue previously authorized commands, such as insulin delivery instructions or sensor calibration updates.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T1557 | Adversary-in-the-Middle | Enterprise |
| T0887 | Wireless Sniffing | ICS |

NIST SP 800-53 Rev. 5 Controls: SC-23, AU-2, SC-13, SC-8

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-018 | Cryptographic Controls | 2 (Direct) | Cybersec. Appendix 1-C |
| CYB-019 | Code/Data/Execution Integrity | 2 (Direct) | Cybersec. Appendix 1-D |
| CYB-014 | Secure Communication Protocols | 1 (Indirect) | Cybersec. Section VI-C |
| CYB-021 | Event Detection and Logging | 1 (Indirect) | Cybersec. Appendix 1-F |

Priority: S=4, L=4, D=3, RPN=48, C=2 → Q1

T-021: RF Jamming/Interference

Category: Communication (COM)  |  Severity: High  |  Primary Layer: 2

Disrupting wireless BLE or Wi-Fi communications between CGM components through radio frequency jamming, causing loss of real-time glucose data transmission.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T0814 | Denial of Service | ICS |
| T1498 | Network Denial of Service | Enterprise |

NIST SP 800-53 Rev. 5 Controls: SC-5, CP-7, SC-40, PE-18

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-022 | Resiliency and Recovery | 2 (Direct) | Cybersec. Appendix 1-G |
| CYB-014 | Secure Communication Protocols | 2 (Direct) | Cybersec. Section VI-C |
| CYB-009 | Defense-in-Depth Architecture | 1 (Indirect) | Cybersec. Section V-D |

Priority: S=3, L=3, D=2, RPN=18, C=4 → Q4

T-022: DNS/Cloud Redirect

Category: Communication (COM)  |  Severity: Medium  |  Primary Layer: 2

Redirecting cloud communications to malicious servers through DNS spoofing or ARP poisoning, enabling interception of patient data or injection of false analytics.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T1071 | Application Layer Protocol | Enterprise |
| T1584 | Compromise Infrastructure | Enterprise |

NIST SP 800-53 Rev. 5 Controls: SC-20, SC-21, SC-22, SI-3

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-018 | Cryptographic Controls | 2 (Direct) | Cybersec. Appendix 1-C |
| CYB-014 | Secure Communication Protocols | 2 (Direct) | Cybersec. Section VI-C |
| CYB-019 | Code/Data/Execution Integrity | 1 (Indirect) | Cybersec. Appendix 1-D |
| CYB-020 | Data Confidentiality and Privacy | 1 (Indirect) | Cybersec. Appendix 1-E |

Priority: S=3, L=2, D=3, RPN=18, C=3 → Q3

T-023: Cloud Infrastructure Compromise

Category: Communication (COM)  |  Severity: High  |  Primary Layer: 2

Compromising the cloud backend services that process CGM analytics, store patient data, and host AI/ML inference models for glucose prediction.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| T1190 | Exploit Public-Facing Application | Enterprise |
| T0883 | Unauthorized Access | ICS |

NIST SP 800-53 Rev. 5 Controls: SC-7, AC-4, SI-3, RA-5, CP-2

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| CYB-016 | Authentication Controls | 2 (Direct) | Cybersec. Appendix 1-A |
| CYB-022 | Resiliency and Recovery | 2 (Direct) | Cybersec. Appendix 1-G |
| CYB-024 | Cybersecurity Testing Evidence | 2 (Direct) | Cybersec. Section VII-A |
| CYB-020 | Data Confidentiality and Privacy | 2 (Direct) | Cybersec. Appendix 1-E |
| CYB-009 | Defense-in-Depth Architecture | 1 (Indirect) | Cybersec. Section V-D |

Priority: S=4, L=3, D=4, RPN=48, C=4 → Q2

1.5. AI-Specific Threats (4)

T-024: Training Data Poisoning

Category: AI-Specific (AI)  |  Severity: Critical  |  Primary Layer: 4

Corrupting training datasets for glucose prediction models by injecting malicious data points, causing systematically biased predictions and unsafe insulin dosing recommendations.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| AML.T0020 | Poison Training Data | ATLAS |

NIST SP 800-53 Rev. 5 Controls: CA-2, CA-8, AU-2, SA-11, SI-7

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| AI-003 | AI Risk Assessment | 2 (Direct) | AI-DSF Section VII |
| AI-004 | AI Data Management | 2 (Direct) | AI-DSF Section VIII |
| AI-002 | AI Training Data Documentation | 2 (Direct) | AI-DSF Section VI |
| AI-005 | AI Performance Validation | 1 (Indirect) | AI-DSF Section IX |
| CYB-006 | Security Risk Assessment with Traceability | 1 (Indirect) | Cybersec. Section V-A |

Priority: S=5, L=3, D=5, RPN=75, C=4 → Q2

T-025: Adversarial Evasion

Category: AI-Specific (AI)  |  Severity: Critical  |  Primary Layer: 4

Crafting perturbations to glucose sensor inputs or model features that cause the AI prediction model to produce incorrect glucose forecasts while appearing normal to monitoring systems.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| AML.T0043 | Craft Adversarial Data | ATLAS |

NIST SP 800-53 Rev. 5 Controls: SI-7, CA-8, SA-11, SI-4

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| AI-003 | AI Risk Assessment | 2 (Direct) | AI-DSF Section VII |
| AI-005 | AI Performance Validation | 2 (Direct) | AI-DSF Section IX |
| CYB-006 | Security Risk Assessment with Traceability | 2 (Direct) | Cybersec. Section V-A |
| CYB-024 | Cybersecurity Testing Evidence | 2 (Direct) | Cybersec. Section VII-A |

Priority: S=5, L=3, D=5, RPN=75, C=4 → Q2

T-026: Model Inversion/Extraction

Category: AI-Specific (AI)  |  Severity: Medium  |  Primary Layer: 4

Extracting proprietary model parameters or inferring private patient training data from the glucose prediction model through repeated queries or side-channel analysis.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| AML.T0024 | Infer Training Data Membership | ATLAS |
| AML.T0044 | Full ML Model Access | ATLAS |

NIST SP 800-53 Rev. 5 Controls: SC-28, AC-3, MP-4, SA-8

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| AI-003 | AI Risk Assessment | 2 (Direct) | AI-DSF Section VII |
| AI-004 | AI Data Management | 1 (Indirect) | AI-DSF Section VIII |
| CYB-020 | Data Confidentiality and Privacy | 2 (Direct) | Cybersec. Appendix 1-E |
| AI-001 | AI/ML Algorithm Transparency | 1 (Indirect) | AI-DSF Section V |

Priority: S=2, L=2, D=4, RPN=16, C=4 → Q4

T-027: Concept Drift

Category: AI-Specific (AI)  |  Severity: High  |  Primary Layer: 4

Gradual degradation of glucose prediction model performance due to changes in patient physiology, sensor characteristics, or data distribution over time, leading to increasingly inaccurate predictions.

MITRE ATT&CK Techniques:

| Technique ID | Technique Name | Matrix |
| --- | --- | --- |
| AML.T0043 | Craft Adversarial Data | ATLAS |
| AML.T0031 | Erode ML Model Integrity | ATLAS |

NIST SP 800-53 Rev. 5 Controls: SI-4, CA-7, SA-11, PM-14

FDA Requirement Mappings:

| Req ID | Requirement | Score | Source |
| --- | --- | --- | --- |
| AI-007 | AI Performance Monitoring | 2 (Direct) | AI-DSF Section XI |
| AI-003 | AI Risk Assessment | 2 (Direct) | AI-DSF Section VII |
| AI-006 | AI Monitoring and Drift Detection | 2 (Direct) | AI-DSF Section X |
| AI-008 | Predetermined Change Control Plan (PCCP) | 1 (Indirect) | AI-DSF Section XII |
| CYB-028 | Postmarket Cybersecurity Monitoring | 1 (Indirect) | Cybersec. Section VIII-B |

Priority: S=4, L=4, D=4, RPN=64, C=4 → Q2

2. Complete FDA Requirement Catalog (38 Requirements)

2.1. FDORA 524B Statutory (Mandatory)

CYB-011: Software Bill of Materials (SBOM) — FDORA 524B(b)(3)

Manufacturers must provide an SBOM in machine-readable format (SPDX or CycloneDX) including commercial, open-source, and off-the-shelf components with version, supplier, and support status.

CYB-023: Vulnerability Patching Capability — FDORA 524B(b)(2)

Devices must support postmarket software/firmware updates and patches to address cybersecurity vulnerabilities on a reasonably justified regular cycle.

CYB-026: Cybersecurity Management Plan — FDORA 524B(b)(1)

Submission must include a plan addressing postmarket cybersecurity monitoring, vulnerability management, coordinated disclosure, and incident response.

2.2. Premarket Cybersecurity Guidance (Recommended)

CYB-001: Threat Modeling — Cybersec. Section IV

Systematic identification of threats using structured methodologies (STRIDE, DREAD, attack trees) appropriate to device architecture and intended use.

CYB-002: Secure Product Development Framework (SPDF) — Cybersec. Section IV-A-1

Integration of cybersecurity activities throughout the product development lifecycle, from concept to decommissioning.

CYB-003: Security Architecture Documentation — Cybersec. Section IV-B

Comprehensive documentation of security architecture including data flow diagrams, trust boundaries, and cryptographic design decisions.

CYB-004: Trust Boundary Definition — Cybersec. Section IV-C

Explicit identification and documentation of all trust boundaries between system components (sensor, controller, pump, cloud, clinician).

CYB-005: Third-Party Component Security — Cybersec. Section IV-D

Security assessment and ongoing monitoring of all third-party software, firmware, and hardware components integrated into the device.

CYB-006: Security Risk Assessment with Traceability — Cybersec. Section V-A

Risk assessment mapping each identified threat to security controls with documented rationale and traceability throughout the TPLC.

CYB-007: Risk-Benefit Analysis — Cybersec. Section V-B

Analysis demonstrating that cybersecurity risks are mitigated to an acceptable level relative to device benefits and intended use.

CYB-008: Security by Design Principles — Cybersec. Section V-C

Application of defense-in-depth, least privilege, fail-secure, and economy of mechanism principles from initial design through deployment.

CYB-009: Defense-in-Depth Architecture — Cybersec. Section V-D

Multiple independent layers of security controls ensuring no single point of failure compromises overall device security.

CYB-010: Least Privilege Implementation — Cybersec. Section V-E

Users, processes, and system components operate with only the minimum privileges necessary for their intended function.

CYB-016: Authentication Controls — Cybersec. Appendix 1-A

Multi-factor authentication for all user and device interactions, including device-to-device, user-to-device, and clinician-to-cloud authentication.

CYB-017: Authorization and Access Controls — Cybersec. Appendix 1-A

Role-based access control (RBAC) policies governing patient, clinician, caregiver, and administrator access to device functions and data.

CYB-018: Cryptographic Controls — Cybersec. Appendix 1-C

Validated cryptographic algorithms for data at rest and in transit, including TLS 1.3 for cloud communications and AES-CCM for BLE, with documented key lifecycle management.

CYB-019: Code/Data/Execution Integrity — Cybersec. Appendix 1-D

Mechanisms ensuring integrity of firmware, software, and data including secure boot, code signing, and runtime integrity verification.

CYB-020: Data Confidentiality and Privacy — Cybersec. Appendix 1-E

Protection of patient glucose data, device identifiers, and personal health information through encryption, access controls, and data minimization.

CYB-021: Event Detection and Logging — Cybersec. Appendix 1-F

Logging of security-relevant events including authentication attempts, configuration changes, firmware updates, and anomalous device behavior.

CYB-022: Resiliency and Recovery — Cybersec. Appendix 1-G

Device ability to maintain essential safety functions (safe-mode insulin delivery, glucose alerting) under cybersecurity compromise and recover to normal operation.

CYB-012: Secure Firmware Update Mechanism — Cybersec. Section VI-A

Authenticated and integrity-verified over-the-air (OTA) firmware update mechanism with rollback capability and version control.

CYB-013: Update Authentication and Verification — Cybersec. Section VI-B

Cryptographic verification of all software and firmware updates before installation, ensuring updates originate from authorized sources.

CYB-014: Secure Communication Protocols — Cybersec. Section VI-C

Use of validated secure communication protocols (TLS 1.3, BLE Secure Simple Pairing) for all inter-component and external communications.

CYB-015: Physical Security Controls — Cybersec. Section VI-D

Hardware-level protections including tamper detection, tamper evidence, secure element storage, and physical access controls.

CYB-024: Cybersecurity Testing Evidence — Cybersec. Section VII-A

Comprehensive testing documentation including vulnerability scanning, static/dynamic analysis, fuzz testing, and penetration testing results.

CYB-025: Penetration Testing — Cybersec. Section VII-B

Independent third-party penetration testing targeting all attack surfaces including BLE, cloud APIs, mobile applications, and firmware.

CYB-027: Coordinated Vulnerability Disclosure — Cybersec. Section VIII-A

Established process for receiving, evaluating, and responding to cybersecurity vulnerability reports from external researchers.

CYB-028: Postmarket Cybersecurity Monitoring — Cybersec. Section VIII-B

Active monitoring for new cybersecurity threats, vulnerabilities in third-party components, and emerging attack techniques relevant to the device.

CYB-029: Incident Response Plan — Cybersec. Section VIII-C

Documented procedures for responding to cybersecurity incidents, including severity classification, containment, remediation, and regulatory notification.

CYB-030: End-of-Life Planning — Cybersec. Section VIII-D

Plan for device end-of-life including continued security support timelines, data migration, and secure decommissioning procedures.

2.3. AI-DSF Draft Guidance

AI-001: AI/ML Algorithm Transparency — AI-DSF Section V

Documentation of AI/ML algorithm design decisions, model architecture, training methodology, and known limitations for regulatory reviewers.

AI-002: AI Training Data Documentation — AI-DSF Section VI

Comprehensive documentation of training data sources, collection methodology, preprocessing steps, annotation procedures, and demographic representation.

AI-003: AI Risk Assessment — AI-DSF Section VII

Risk assessment specifically addressing AI/ML threats including data poisoning, adversarial inputs, model drift, and algorithmic bias.

AI-004: AI Data Management — AI-DSF Section VIII

Policies and procedures for AI training/validation/test data lifecycle management including provenance tracking, quality assurance, and data integrity.

AI-005: AI Performance Validation — AI-DSF Section IX

Pre-deployment validation of AI model performance across representative patient populations, edge cases, and adversarial conditions.

AI-006: AI Monitoring and Drift Detection — AI-DSF Section X

Ongoing monitoring for model performance degradation, concept drift, and data distribution shifts post-deployment.

AI-007: AI Performance Monitoring — AI-DSF Section XI

Continuous real-world performance monitoring with defined metrics, alert thresholds, and corrective action procedures.

AI-008: Predetermined Change Control Plan (PCCP) — AI-DSF Section XII

Documentation of anticipated AI model modifications, validation requirements for each change type, and conditions requiring new regulatory submission.

3. Implementation Priority Summary

Q1: Quick Wins (5 threats)

| ID | Threat | S | L | D | RPN | C | Layer |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T-006 | Insufficient Authentication | 5 | 5 | 3 | 75 | 2 | 3 |
| T-009 | Replay Attacks | 4 | 4 | 3 | 48 | 2 | 2 |
| T-010 | Denial of Service (DoS) | 4 | 4 | 3 | 48 | 1 | 2 |
| T-019 | BLE Pairing Exploitation | 4 | 4 | 3 | 48 | 3 | 2 |
| T-020 | Wireless Replay | 4 | 4 | 3 | 48 | 2 | 2 |

Q2: Strategic Priorities (7 threats)

| ID | Threat | S | L | D | RPN | C | Layer |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T-024 | Training Data Poisoning | 5 | 3 | 5 | 75 | 4 | 4 |
| T-025 | Adversarial Evasion | 5 | 3 | 5 | 75 | 4 | 4 |
| T-027 | Concept Drift | 4 | 4 | 4 | 64 | 4 | 4 |
| T-007 | Firmware Code Injection | 5 | 3 | 4 | 60 | 4 | 1 |
| T-023 | Cloud Infrastructure Compromise | 4 | 3 | 4 | 48 | 4 | 2 |
| T-002 | MPC Model Inaccuracy | 5 | 3 | 3 | 45 | 4 | 4 |
| T-011 | Man-in-the-Middle (MITM) Attacks | 5 | 3 | 3 | 45 | 4 | 2 |

Q3: Efficiency Gains (8 threats)

| ID | Threat | S | L | D | RPN | C | Layer |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T-005 | Cloud API Exploitation | 4 | 3 | 3 | 36 | 3 | 2 |
| T-008 | Session Hijacking | 4 | 3 | 3 | 36 | 3 | 3 |
| T-012 | Privilege Escalation | 4 | 3 | 3 | 36 | 2 | 3 |
| T-013 | CGM Sensor Failures | 4 | 4 | 2 | 32 | 3 | 1 |
| T-003 | Setpoint Manipulation | 5 | 3 | 2 | 30 | 3 | 3 |
| T-018 | BLE Eavesdropping | 3 | 4 | 2 | 24 | 2 | 2 |
| T-022 | DNS/Cloud Redirect | 3 | 2 | 3 | 18 | 3 | 2 |
| T-015 | Physical Tampering | 3 | 2 | 2 | 12 | 3 | 1 |

Q4: Long-term Initiatives (7 threats)

| ID | Threat | S | L | D | RPN | C | Layer |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T-004 | Control Algorithm Exploitation | 4 | 2 | 4 | 32 | 5 | 4 |
| T-017 | Hardware Supply Chain Compromise | 4 | 2 | 4 | 32 | 5 | 1 |
| T-001 | Glucose-Insulin Model Manipulation | 5 | 2 | 3 | 30 | 4 | 4 |
| T-014 | Sensor Calibration Manipulation | 5 | 2 | 3 | 30 | 4 | 1 |
| T-016 | Battery/Power Depletion | 3 | 3 | 2 | 18 | 4 | 1 |
| T-021 | RF Jamming/Interference | 3 | 3 | 2 | 18 | 4 | 2 |
| T-026 | Model Inversion/Extraction | 2 | 2 | 4 | 16 | 4 | 4 |